README.DOC          News and corrections MS-DOS 3.20          1987-08-10

The contents of this file is an addendum to the previous TeleNova MS-DOS

documentation for the COMPIS/SCANDIS computer. It replaces all previous

README.DOC files.

Topic                                                               Line

------------------------------------------------------------------------

General changes in MS-DOS 3.20                                        36

CITOH           renamed to PREMUL

COM.SYS         Installable device for serial communication           59

COMPMODE        renamed to STDMODE

CPM             Boot CP/M-86                                          63

CPMDIR/         Handle CP/M-86 flexible disk                          66

FROMCPM/

TOCPM

CS              Change character set                                  71

DUMMY.SYS       Installable dummy block device                        77

EMULATE         PC ROM BIOS emulator                                  80

FCOPY/          Prepare and copy flexible disk                       108

FPREP

FORMAT          Prepare MS-DOS disk                                  113

GETKEY          Read key value in batch file                         119

HDISK           Enhanced hard disk driver                            122

KEYBxx          Change keyboard layout                               127

NETPRINT        Print utility                                        151

NEWYEAR         Set default year (for COMPIS/SCANDIS I)              177

NOVAED          Full screen text editor                              182

PBOOT           Boot p-System                                        185

PREMUL          Filter PC characters for 7-bit printers              188

RAM.SYS         Installable device for RAM disks (only renamed)      201

STDMODE         Set standard keyboard codes and character set        203

------------------------------------------------------------------------

General changes in MS-DOS 3.20

Switching between standard/PC-mode does not change the

screen handling ANSI/VT52-mode. Keyboard redefinitions (cf.

KEYDEF) are preserved. Default is standard-mode and ANSI-

mode.

CTRL-X (not ESC) cancels the current input line.

If no network is connected, the devices LPT1 and PRN are

the parallel printer connected to J7 and the device AUX is

serial communication on J2 (9600, none, 8, 1). Redirection

and change of parameters is available with COM.SYS and MODE.

System key combinations (Super)Shift+Ctrl+key are no longer

affected by the state of the CapsLock-key.

Single-drive systems emulates two flexible disk drives (A

and B) with the available disk drive. MS-DOS will prompt for

change of disk.

------------------------------------------------------------------------

COM.SYS     An installable character device for extended serial

communication (cf. MODE). No longer included in the default

CONFIG.SYS.

CPM         Quick boot of CP/M-86 without memory reset. Useful with hard

disk or network.

CPMDIR/     The CP/M-86 flexible disk utility commands are now capable

FROMCPM/    of handling disks with 8 or 9 sectors/track. An optional

TOCPM       last numeric argument specifies a CP/M user number. Default

is user number 0.

CS          The change character set command can be used to change

nationality of the character set also in PC-mode. The normal

PC character set is number 0 and the Danish/Norwegian PC

character set is number 1. In standard-mode the character

set can also be specified in letters (cf. CP/M-86).

DUMMY.SYS   An installable dummy block device. Can be used to obtain the

same device names (i.e. Z:) in different configurations.

EMULATE     EMULATE v3.0 is an enhanced version of the PC ROM BIOS

emulator. Three levels of emulation are available.

Level 1 corresponds to PC-mode.

Level 2 is a slightly enhanced version of EMULATE v2.0.

Level 3, in addition to level 2, supports direct handling of

the video memory for the monochrome or color/graphics

adaptors. This level of emulation requires 768K of RAM

memory.

Level 2 and 3 stays resident in memory when loaded.

An optional parameter /1, /2 (default if 256K) or /3

(default if 768K) specifies the level of emulation. If level

2 or 3 is already loaded, the level of emulation can not be

changed without resetting the computer. This must be done

with the RESET button on the front of the computer.

The different levels of emulation and other options are

described in detail in the file EMULATE.DOC.

NOTE! Use always the RESET button to restart the computer

whenever the emulator is loaded!

NOTE! EMULATE v3.0 may not be used together with PREMUL!

FCOPY/      The general disk utility commands can prepare CP/M-86 and

FPREP       p-system flexible disks, and copy MS-DOS (not 40 tracks),

CP/M-86 and p-system flexible disks. Refer to your CP/M-86

manuals for detailed information.

FORMAT      Single-sided flexible disks must be formated with parameter

/1 (not /L). Parameter /4 (not /I) gives 40 tracks/side (PC

format). For standard disk format in high density drive

give parameter /9 (not /C). With parameter /C the disk

directory is cleared.

GETKEY      A program for reading key value in batch files. The key

value is available in the parameter ERRORLEVEL.

HDISK       Enhanced hard disk driver (cf. Hard Disk Manual). It

replaces the driver in ROM BIOS version 3.02 (or later 3.0x)

and stays resident in memory. HDISK sets the ERRORLEVEL 1 if

"Driver is already loaded" or "Incorrect ROM BIOS version".

KEYBxx      The keyboard commands will activate the change immediately

(no need to run STDMODE or PCMODE afterwards).

The KEYBDA now gives a correct Danish (not Norwegian)

keyboard layout and the new KEYBUS command gives an US

English keyboard layout.

The KEYBDA and KEYBUS commands loads a set of keyboard

tables which replaces some of the keyboard tables resident

in ROM BIOS.

Only one keyboard program should be loaded after booting

MS-DOS. The program remains resident in memory until you

perform a system reset or power off the computer. If you

load a second keyboard program, it will gain control of the

keystrokes you type. However, the earlier program would

still be in memory, and cannot be returned to.

The KEYBDA and KEYBUS commands increase the resident size of

MS-DOS in memory by approximately 1K bytes.

The KEYBGR command gives a correct German keyboard layout

only in PC-mode.

NETPRINT    Alternative print command that will take a filename as

parameter. The file is output to the standard printer (local

parallel or network printer). For network printers the

spoolfile is automatically closed. PC print filter (cf.

PREMUL) for all 7-bit printers or change of character set on

C.Itoh 8510/1550 compatible printers is available.

NETPRINT reads two additional parameters from MS-DOS. They

can be assigned by the SET command.

The parameter PRINTERCLASS specifies which of the 3 possible

network printers to use (1, 2 or 3). Default is

PRINTERCLASS=1.

The parameter PRINTERMODE specifies print conversion or

C.Itoh character set. The following options are available:

0      No conversion (default)

PC     PC to 7-bit character conversion

JA, FR, US, UK, GE, SW, IT, SP, DN, NOR, NET, AF, FR2,

or UK2    Change C.Itoh character set on printer

If you put the line

SET PRINTERMODE=PC

into your AUTOEXEC.BAT file, you will get PC character

conversion.

NEWYEAR     The IO.SYS file contains a default year, used for computers

with old clock chips (COMPIS I and SCANDIS I). The NEWYEAR

program updates this default year on the disk present in the

default drive.

NOVAED      The full screen text editor uses the currently active

character set (standard- or PC-mode).

PBOOT       Quick boot of p-System without memory reset (cf. Hard Disk

Manual). Useful with hard disk or network.

PREMUL      The PC printer filter can be used for all 7-bit local

parallel printers.

It is possible to enable or disable the printer filter by

supplying an optional parameter: /ON or /+ to enable, /OFF

or /- to disable. The parameters /VER and /? display version

number and filter status respectively.

The printer filter is disabled during graphic screen dump.

NOTE! PREMUL may not be used together with EMULATE v2.0 or

later!

RAM.SYS     An installable block device for RAM disks.

STDMODE     Sets the standard COMPIS/SCANDIS keyboard codes and

character set (cf. CP/M-86). Does not change the current

ANSI/VT52 screen handling modes.